# COMPLIANCE MEMORANDUM

| TO: | Terminal Operations Review Board |
| --- | --- |
| FROM: | Safety & Compliance Department |
| DATE: | 29 March 2026 |
| SUBJECT: | IMO Circular Compliance Analysis and Automated Inspection System Implementation |

# 1. EXECUTIVE SUMMARY

This memorandum presents findings from a comprehensive cross-reference analysis of IMO safety circulars against vessel operational history for OPS-LISBON-VESSEL-908 (ATLANTIC PIONEER). The analysis covers the period from March 2024 through February 2026 and identifies critical compliance gaps requiring immediate remediation.

KEY FINDINGS:

• 20 safety incidents recorded over 19 voyages (63% voyage affect rate)
• 14 incidents (70%) directly addressable by new IMO circulars
• 5 compliance gaps require immediate action before next voyage
• €642,000 invested in circular-driven maintenance (65% of total spend)
• Estimated remediation cost for remaining gaps: €185,000 - €240,000

CRITICAL COMPLIANCE GAPS IDENTIFIED:

1. Gas monitor calibration expired (INC-908-2026-002) - OPEN status, 2 weeks overdue
2. IMSBC 08-25 voluntary application not followed (INC-908-2026-001)
3. Fumigation contractor procedures not updated per MSC.1/Circ.1264/Rev.1
4. Cargo Securing Manual amendment for lashing software v5.0 not verified
5. Dangerous goods shipper documentation gaps (3 incidents)

This memo recommends implementation of an automated Container Loading Flag System to prevent recurrence of documented incidents through real-time trigger-based inspection protocols.

# 2. BACKGROUND

The International Maritime Organization (IMO) Maritime Safety Committee (MSC 110, June 2025) and Sub-Committee on Carriage of Cargoes and Containers (CCC 10, September 2024) issued significant safety circulars affecting cargo operations, dangerous goods handling, and vessel safety systems.

Vessel OPS-LISBON-VESSEL-908 (ATLANTIC PIONEER), an 8,500 TEU container ship operated by EuroContainer Lines, has been selected for detailed compliance analysis based on:

• High voyage frequency (19 voyages in 24 months)
• Diverse cargo portfolio (containers, bulk, dangerous goods, reefer)
• Multiple port state control regimes (Paris MoU, Tokyo MoU, Latin America MoU)
• Documented incident history across multiple safety categories

The vessel's operational history provides a representative sample for evaluating circular implementation effectiveness and identifying systemic compliance gaps.

# 3. DETAILED FINDINGS

## 3.1 IMSBC Code Amendments 08-25

Entry into Force: 1 January 2027 (voluntary from 1 January 2026)

Related Incidents: 4

| Incident ID | Date | Issue | Status |
| --- | --- | --- | --- |
| INC-908-2024-007 | 2024-11-03 | DRI under old IMSBC schedule | Voluntary Apply |
| INC-908-2025-005 | 2025-06-28 | Fish meal antioxidant undeclared | Voluntary Apply |
| INC-908-2026-001 | 2026-01-15 | Zinc slag under generic schedule | MANDATORY |
| INC-908-2024-008 | 2024-12-18 | Lashing software discrepancy | Voluntary Apply |

COMPLIANCE GAP: Zinc slag incident (INC-908-2026-001) occurred AFTER voluntary application date of 1 January 2026. This constitutes a reportable deficiency if discovered during Port State Control inspection.

## 3.2 MSC.1/Circ.1264/Rev.1 - Fumigation Safety

Status: Approved MSC 110 (June 2025)

Related Incidents: 3

| Incident ID | Date | Issue | Status |
| --- | --- | --- | --- |
| INC-908-2024-002 | 2024-04-22 | Phosphine gas detection (12 ppm) | Procedural Gap |
| INC-908-2025-003 | 2025-03-30 | Fumigation warning sign missing | Procedural Gap |
| INC-908-2024-007 | 2024-11-03 | DRI cargo - phosphine fire risk | Procedural Gap |

COMPLIANCE GAP: Fumigation contractor vetting procedure not updated per MSC.1/Circ.1264/Rev.1. Next fumigation operation carries elevated risk.

## 3.3 Revised A.1050(27) - Enclosed Space Entry

Status: ADOPTED MSC 110 (June 2025)

Related Incidents: 2

| Incident ID | Date | Issue | Status |
| --- | --- | --- | --- |
| INC-908-2024-006 | 2024-09-25 | Oxygen deficiency (18.5%) | Partially Resolved |
| INC-908-2026-002 | 2026-02-08 | Gas monitor calibration expired | OPEN GAP |

CRITICAL COMPLIANCE GAP: Gas monitor calibration expired 2 weeks ago (INC-908-2026-002). Status still "Open". This is a detainable deficiency under Port State Control inspection. Revised A.1050(27) explicitly requires calibrated equipment for enclosed space entry.

## 3.4 MSC.1/Circ.1353/Rev.2 - Cargo Securing Manual

Status: Revision initiated CCC 10 (September 2024)

Related Incidents: 5 (highest frequency category - 26% incident rate)

COMPLIANCE GAP: Lashing software discrepancy (INC-908-2024-008) was "corrected" but CSM formal amendment for software v5.0 not verified with classification society. This is a common Port State Control deficiency.

## 3.5 IMDG Code Column 17 Amendments

Expected Adoption: MSC 111 (2026)

Related Incidents: 3

| Incident ID | Date | Issue | Status |
| --- | --- | --- | --- |
| INC-908-2024-003 | 2024-05-08 | Undeclared lithium batteries | Shipper Gap |
| INC-908-2025-001 | 2025-01-22 | Column 17 information missing | Shipper Gap |
| INC-908-2025-009 | 2025-11-30 | Lithium battery thermal runaway | Shipper Gap |

# 4. COMPLIANCE GAP SUMMARY

| Priority | Gap | Circular | Incident | Remediation |
| --- | --- | --- | --- | --- |
| CRITICAL | Gas monitor calibration expired | A.1050(27) | INC-908-2026-002 | Verify recalibration; update tracking |
| CRITICAL | IMSBC voluntary not applied | IMSBC 08-25 | INC-908-2026-001 | Update cargo acceptance checklist |
| HIGH | Fumigation procedures outdated | MSC.1/Circ.1264/Rev.1 | INC-908-2024-002 | Update contractor vetting |
| HIGH | CSM amendment not verified | MSC.1/Circ.1353/Rev.2 | INC-908-2024-008 | Confirm class approval |
| MEDIUM | DG shipper documentation gaps | IMDG 43-26 | Multiple | Create approved shipper list |

# 5. PROPOSED AUTOMATED WORKFLOW: CONTAINER LOADING FLAG SYSTEM

To prevent recurrence of documented incidents and ensure real-time IMO circular compliance, we propose implementation of an automated Container Loading Flag System. This system integrates with existing Terminal Operating Systems (TOS) and activates manual inspection protocols based on predefined trigger conditions.

SYSTEM ARCHITECTURE:

The Flag System operates as a middleware layer between the Terminal Operating System and loading operations, evaluating each container against trigger conditions before loading approval is granted.

TRIGGER CATEGORIES:

The system monitors seven trigger categories corresponding to IMO circular requirements and historical incident patterns:

| Category | Trigger Count | Priority Level |
| --- | --- | --- |
| Dangerous Goods (DG) | 4 triggers | CRITICAL |
| Bulk Cargo (BC) | 5 triggers | HIGH |
| Fumigation (FUM) | 4 triggers | CRITICAL |
| Cargo Securing (CS) | 5 triggers | HIGH |
| Enclosed Space (ES) | 4 triggers | CRITICAL |
| Pilot Transfer (PT) | 3 triggers | HIGH |
| Vessel History (VH) | 3 triggers | MEDIUM |

## 5.1 EXACT TRIGGER CONDITIONS

The following trigger conditions activate manual inspection requirements. Each trigger is evaluated in real-time during container processing:

### Dangerous Goods Triggers

| ID | Trigger Name | Condition | Action | Approval |
| --- | --- | --- | --- | --- |
| DG-001 | Missing IMDG Column 17 Data | shippingDocument.column17 == null OR column17.flashpoint == null OR column17.toxicityLevel == null | HOLD_LOADING | Chief Officer |
| DG-002 | Lithium Battery Shipment | cargo.unNumber IN [UN3480, UN3481, UN3090, UN3091] AND packagingGroup != VERIFIED | ENHANCED_INSPECTION | Chief Officer |
| DG-003 | Undeclared DG Detection | container.xrayAnomaly == true OR container.sniffTestPositive == true | QUARANTINE | Master |
| DG-004 | DG Shipper Not Approved | shipper.dgComplianceRating == null OR shipper.dgComplianceRating < 0.85 | ENHANCED_DOCUMENT_CHECK | Chief Officer |

### Bulk Cargo Triggers

| ID | Trigger Name | Condition | Action | Approval |
| --- | --- | --- | --- | --- |
| BC-001 | IMSBC Schedule Mismatch | cargo.imsbcSchedule != CURRENT_08-25 OR == LEGACY | HOLD_LOADING | Chief Officer |
| BC-002 | DRI Moisture Certification | cargo.bcsnCode IN [DRI-A, DRI-B] AND moistureContent.certified == false | HOLD_LOADING | Chief Officer |
| BC-003 | Fish Meal Self-Heating | cargo.bcsnCode CONTAINS FISH MEAL AND antioxidantTreatment.declared == false | HOLD_LOADING | Master |
| BC-004 | New IMSBC Schedule Cargo | cargo.bcsnCode IN [Zinc Slag, Iron Ore Briquettes, etc.] | ENHANCED_INSPECTION | Chief Officer |
| BC-005 | Temperature Anomaly | cargo.temperature > 55°C OR temperatureRising == true | QUARANTINE | Master |

### Fumigation Triggers

| ID | Trigger Name | Condition | Action | Approval |
| --- | --- | --- | --- | --- |
| FUM-001 | Phosphine Without Seal Test | fumigantType == PHOSPHINE AND sealIntegrityTest != PASSED | HOLD_LOADING | Master |
| FUM-002 | Missing Warning Signs | container.fumigated == true AND warningSignsApplied == false | HOLD_LOADING | Chief Officer |
| FUM-003 | Gas Level Anomaly | phosphineLevel > 0.1 ppm OR oxygenLevel < 19.5% | EVACUATE_AND_VENTILATE | Master |
| FUM-004 | Contractor Not Approved | contractor.approved == false OR lastAudit < 2025-06-01 | ENHANCED_VERIFICATION | Chief Officer |

## 5.2 ACTION DEFINITIONS

| Action | Description | Escalation Time |
| --- | --- | --- |
| HOLD_LOADING | Stop loading. Container cannot be loaded until inspection completed. | 30 minutes |
| ENHANCED_INSPECTION | Enhanced inspection required. Loading may continue with inspection. | 1 hour |
| QUARANTINE | Isolate container in quarantine area. Master approval required. | 15 minutes |
| EVACUATE_AND_VENTILATE | Immediate evacuation. Activate emergency ventilation. | IMMEDIATE |
| SUSPEND_OPERATIONS | Suspend all operations until equipment verified. | IMMEDIATE |

# 6. IMPLEMENTATION TIMELINE

PHASE 1 - IMMEDIATE (Within 48 hours):
• Verify gas monitor calibration certificates (INC-908-2026-002)
• Update cargo acceptance checklist for IMSBC 08-25
• Suspend enclosed space operations until calibration verified

PHASE 2 - SHORT TERM (Within 7 days):
• Update fumigation contractor procedures per MSC.1/Circ.1264/Rev.1
• Verify CSM amendment for lashing software v5.0 with classification society
• Deploy Flag System configuration to test environment

PHASE 3 - MEDIUM TERM (Within 30 days):
• Create DG shipper approved list with compliance ratings
• Implement calibration tracking system with automated alerts
• Complete Flag System integration with Terminal Operating System
• Conduct crew training on trigger response procedures

PHASE 4 - LONG TERM (Within 90 days):
• Full Flag System production deployment
• Monthly statistics reporting implementation
• Port State Control deficiency rate monitoring
• Quarterly system effectiveness review

# 7. RECOMMENDATIONS

1. APPROVE immediate implementation of the Container Loading Flag System as specified in the attached configuration file (container_loading_flag_system_config.json).

2. AUTHORIZE emergency recalibration of all gas monitoring equipment and suspension of enclosed space operations until certificates verified.

3. DIRECT Operations Department to update cargo acceptance procedures to require IMSBC 08-25 schedules for all bulk cargo, effective immediately.

4. MANDATE Safety Department to revise fumigation contractor vetting procedures per MSC.1/Circ.1264/Rev.1 before next fumigation operation.

5. REQUIRE Technical Superintendent to obtain classification society approval letter for CSM amendment incorporating lashing software v5.0 parameters.

6. ESTABLISH monthly Flag System statistics reporting to Terminal Operations Review Board, including trigger counts, resolution times, and repeat offender analysis.

7. ALLOCATE budget for Flag System integration with existing Terminal Operating System and gas monitoring infrastructure.

# 8. ATTACHMENTS

1. IMO Circular Cross-Reference Analysis (imo_circular_vessel_cross_reference_analysis.md)

2. Vessel Operational History (vessel_OPS-LISBON-VESSEL-908_operational_history.json)

3. Container Loading Flag System Configuration (container_loading_flag_system_config.json)

4. IMO Safety Circulars Summary (imo_safety_circulars_2026-03-29.md)

____________________________________________________________

Prepared by: Safety & Compliance Department

Date: 29 March 2026

For questions regarding this memorandum, contact:

Safety Department | safety@eurocontainerlines.com